     C-150G  NORMAL CHECKLIST  -  N2942J                               

BEFORE STARTING ENGINE    
 Preflight Inspection  –  Complete
 Flaps  –  Up
 Logbook  –  Checked

 Circuit Breakers  –  Checked 

 Avionics Master  –  OFF

 Seats, Seat Belts  –  Locked, ON
 Passenger Briefing  –  Completed

 Fuel Valve Handle  –  ON

 Prime  –  As Req’d

 Master Switch  –  ON

 Ignition Switch  –  BOTH

 Carb Heat  –  Cold

 Throttle  –  Open ¼”

 Mixture  –  Rich

 Beacon  –  ON

 Brakes  –  Test and hold 

 Prop Area  –  Clear

 Starter Handle  –  Pull

AFTER START

 Throttle Set  –  1000 RPM

 Oil Pressure  –  Check

 Mixture  –  Lean ½ way

 Avionics Master  –  ON

 Ammeter  –  Charging

 Radios, GPS  –  ON
 Flight Instruments:  HDG, ALT  –  Set

   [ATIS-Clearance-Taxi Clearance]

 Brakes  –  Test and release

BEFORE TAKEOFF
 Mixture  –  Rich

 Throttle  –  1700 RPM

 Engine Instruments  –  Green Arc

 Magnetos  –  Check (Max 75 diff.)

 Carb Heat  –  Check

 Suction Gage  –  Check

 Throttle  –  1000 RPM
 Mixture  –  Lean ½ way

 Flight Controls  –  Check

 Trim Tab  –  TAKE-OFF setting

 Cabin Doors, Windows  –  Latched

 Flight Instruments:  HDG, ALT  –  Check
 NAV Radios, GPS  –  Set
TAKING THE RUNWAY           

  Mixture  –  Rich

  Lights  –  ON  
  Transponder  –  ON / ALT

  Time – Note

TAKEOFF

  Normal  –  Flaps Up


Climb Vy  –  72 MPH

  Max. Performance –  Flaps Up


Climb Vx  –  64 MPH

CLIMB

  Normal  –  75 – 80 MPH
   Lights  –  As desired
CRUISE
  Power  –  2000 – 2750 (pg. 5-4)

  Mixture  –  Lean

DESCENT / APPROACH 

  Mixture  –  Enrichen

  Lights  –  ON

  Seat Backs  –  Up
  Seat Belts  – ON

BEFORE LANDING

  Mixture  –  Rich

  Carb Heat  –  ON

  Flaps  –  As desired < 100 MPH

AFTER LANDING

  Flaps  –  Up

  Carb Heat  –  Cold

  Lights  –  As desired

  Transponder  –  STBY
  Mixture  –  Lean ½ way

SHUTDOWN / SECURING
     [ Close Flight Plan ]

  Avionics Master  –  OFF
  All Lights except Beacon – OFF
  Mixture  –  Idle cut-off

  Mags  –  OFF

  Master switch  –  OFF

  Control Lock(s)  –  Installed

v.1.4   5/1/2010


