Briefing notes

Localizer back course approach
Rev. 1/24/08
· Fly in TRK or HDG mode, do NOT use LOC ONLY. 

(Reverse sensing – won’t work)

· Set freq. (ICDB) and Front course / 144° in NAV / RAD

· Hard tune CDB VOR for DME and Missed approach

· PF uses “APPR” on ND  [draw]

· PM uses “MAP” on ND

· Disregard LOC scale on PFD

· NPA shall be autocoupled. If <1,000’ or <3 must be monitored

(7-1-5-7;  6.43)

-
Use V/S

-
Challenging since manipulating both vertical and lateral modes
-
Challenging since LOC antenna is at approach end – more sensitive

PACD LLZ Back CRS 32  - unique issues
· Read all notes carefully

· Steep approach:  3.4° = 970’/min. descent,  Brief greater sink rate

· Suggest using LIDO altitudes
· 1220’ crossing restriction very steep: 380’/mile to rwy.

· MAP / VDP / MDA all at 6.2 DME

· 3-bar VASI: [draw]

Not in database. Approach Building (technique):

· Select 32 (at bottom of list)
· Manually add WPTs and altitudes 

· Add a DISCON and CDB VOR for missed approach
