Briefing notes

HYD 1 & 2 FAIL with HYD 3 ELEV OFF

“Sioux City” scenario
· Presentation
· Checklists: 

· HYD 1 & 2 FAIL (with HYD 3 ELEV OFF)
· Also “Engine Fire / Severe Damage” for #2 (briskly)

· Technique

· Initial loss of #2 causes pitch up.  Takes a while to realize that elevators are inop

· 0°/EXT approach – so flat sight picture and fast: ~190K at 440,000#.

· Trim=speed ;   Power=approach path / altitude

· Consider one pilot on yoke for lateral control, other on throttles

